| Activity/Work Task: Working At Heights | Overall Risk Assessment Code (RAC) (Use highest code) |  |  |  |  |  |  |  | M |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Project Location: | Risk Assessment Code (RAC) Matrix |  |  |  |  |  |  |  |  |  |
| Contract Number: | Severity | Probability |  |  |  |  |  |  |  |  |
| Date Prepared: |  | Frequent (F) | Likely (L) | Occasional (O) |  |  |  | Seldom (S) |  | Unlikely (U) |
| Prepared by (Name/Title): | Catastrophic (C) | E | E |  | H |  | H |  |  | M |
|  | Critical (Cr) | E | H |  | H |  | M |  |  | L |
| Reviewed by (Name/Title): | Marginal (M) | H | M |  | M |  | L |  |  | L |
|  | Negligible (N) | M | L |  | L |  | L |  |  | L |
| Employer/GBU: | Step 1: Review each “Hazard” with identified safety “Controls” and determine RAC (See above). The RAC is developed after correctly identifying all the hazards and fully implementing all controls. |  |  |  |  |  |  |  |  |  |
| Notes: (Field Notes, Review Comments, etc.) References : | P “Probability” is the likelihood to cause an incident, near miss, or accident and identified as: Frequent (F), Likely (L), Occasional (O), Seldom (S) or Unlikely (U). |  |  |  |  | RAC Chart |  |  |  |  |
|  | S “Severity” is the outcome/degree if an incident, near miss, or accident did occur and identified as: Catastrophic (C), Critical (Cr), Marginal (M), or Negligible (N) |  |  |  |  | E = Extremely High Risk |  |  |  |  |
|  |  |  |  |  |  | H = High Risk |  |  |  |  |
|  | Step 2: Identify the RAC (Probability/Severity) as E, H, M, or L for each “Hazard” on AHA. Annotate the overall highest RAC at the top of AHA. |  |  |  |  | M = Moderate Risk |  |  |  |  |
|  |  |  |  |  |  | L = Low Risk |  |  |  |  |

| Job Steps | Hazards | Controls | P | S | RAC |
| --- | --- | --- | --- | --- | --- |
| - 1. Movement across the Site. | - Falls on level. | 1.1.1 Maintain a clear designated walkway. Avoid protruding parts of the material. Avoid stepping over obstacles such as tools, materials and cables. 1.1.2 Correct safety footwear should be worn. 1.1.3 No oil, grease, and other slippery substance on floors and shoes. Avoid slippery walkway surface. 1.1.4 Maintain adequate illumination/lighting 1.1.5 Follow mandatory signs. | S | M | L |
|  | 1.2 Exposure to sharp and protruding materials. | 1.2.1 Attend site orientation / pre task briefing prior to start activity. 1.2.2 Maintain adequate illumination/lighting 1.2.4 Follow mandatory signs. 1.2.5 Make sure that sharp objects are covered/insulated. | S | Cr | M |
|  | 1.3 Improperly supported stack of materials, too high stack, improper arranged materials. | 1.3.1 Make sure that contractor complies with the design and standards of material storage. 1.3.2 Verify that safety signs and barricades are provided. 1.3.3 Follow mandatory safety signs. 1.3.4 Verify that contractor uses suitable storage racks intended for the materials being stored. 1.3.5 Make sure that contractor practice safe arrangement and storage of materials. 1.3.6 Maintain safe distance from unstable and poorly supported stack of materials. 1.3.7 Follow mandatory signs. 1.3.8 Wear approved head and eye protection. 1.3.9 Maintain no unsecured materials are placed overhead. | S | Cr | M |
| 2. Inspection on elevated surfaces or working platform above 1.8 Meters. - Pre inspection before accessing scaffold. | 2.1 Falling material. | 2.1.1 Comply with Site PPE Regulation (Hard hat, Steel toe safety shoes & eye protection). 2.1.2 Verify edge protection is in place. 2.1.3 Verify no unsecured materials above. Barricades/signage below work area. | U | M | L |
|  | 2.2 Unsafe scaffold | 2.2.1 Verify scaffold is green tagged prior to use. 2.2.2 Verify safe working load of the scaffold is Displayed and adhered to. 2.2.3 Verify scaffold is placed on correct feet and base plates 2.2.4 Verify scaffold is secured/ tied off if required. | S | M | L |
| - Pre inspection before accessing scaffold. (Cont…) |  | 2.2.5 Verify wheels of the mobile tower scaffold are locked. 2.2.6 Verify scaffold is placed in firm and level ground. | S | M | L |
| - Climbing up the scaffold through stairway / ladder. | 2.3 Failure of the stairway / ladder. | 2.3.1 Verify safe position of stairs. 2.3.2 Install staircase at least 30 degrees, and no more than 50 degrees, from the horizontal. 2.3.3 Install ladder at 75 degree angle (1:4 ratios). 2.3.4 Verify ladder / staircase is secured. 2.3.5 Verify ladder/stairs is on firm base and footing. 2.3.6 Verify railing of stairways as per standard. 2.3.7 Only one person at a time will use the ladder. 2.3.8 Do not use job made ladder unless certified. 2.3.9 Check SWL of the ladder. | S | M | L |
|  | 2.4 Falling from height | 2.4.1 Always maintain 3 point contact while using ladder. 2.4.2 Verify stairs are clear from any obstruction & non slippery. 2.4.3 Verify rungs of the ladder are clear of mud/grease & are not damaged. 2.4.4 Verify proper footwear is worn and is free from mud/grease. 2.4.5 Verify ladder extends 1 meter from the top landing platform. 2.4.6 Verify ladder is safely positioned at 75 degree angle (1:4 ratio). 2.4.7 Do not carry a load on a ladder 3.2.8 Verify that adequately illumination is provided on site. 2.4.9 Always face the ladder when climbing up or down. | U | Cr | L |
| -Failure of the stairway / ladder. | 2.5 Failure of scaffold. | 2.5.1 Verify scaffold is green tagged prior to use. 2.5.2 Verify safe working load of the scaffold is Displayed and adhered to. 2.5.3 Verify scaffold is placed on correct feet and base plates. 2.5.4 Verify scaffold is secured/ tied off if required. 2.5.5 Verify wheels of the mobile tower scaffold are locked. 2.5.6 Verify scaffold is placed in firm ground. | S | M | L |
|  | 2.6 Fall on the same level. | 2.6.1 Verify scaffold platform is free from any obstruction. 2.6.2 Verify scaffold platform is free from oil, grease & mud. 2.6.3 Always look and scan ahead of your route. 2.6.4 Hands should always be out of the pocket. 2.6.5 Avoid stepping over cables, tools, materials, etc. 2.6.6 Verify platform planks are secured and extends 12” beyond its support/ledger | U | M | L |
|  | 2.7Falling materials. | 2.7.1 Comply with Site PPE Regulation (Hard hat, Steel toe safety shoes & eye protection). 2.7.2 Verify toe board in place. 2.7.3 Verify no unsecured materials above. Barricades/signage below work area. | U | M | L |
|  | 2.8Protruding materials. | 2.8.1 Always look and scan ahead of your route 2.8.2 Always follow mandatory warning signage 2.8.3 Verify that adequately illumination is provided on site. 2.8.4 Verify all protruding materials are provided with end caps. | U | M | L |
|  | 2.9Falling from height. | 2.9.1 Verify edge/fall protection are in place. 2.9.2 While working on scaffold Verify no alteration to take place of any scaffold components, handrails, mid rails, toe boards, bracing, ladder, frames, platform, couplers or ties. 2.9.3 Verify no large gaps in between the platform planks or between the platform and working surface. 2.9.4 Verify platform planks are secured and extends 12” beyond its supports/ledgers 2.9.5 Do not over reach beyond the guardrails 2.9.6 Never lean on guard rails. | U | Cr | L |
| -Climbing down from the scaffold through stairway/ladder. | 2.10 Falling from height. | 2.10.1 Always maintain 3 point contact while using ladder. 2.10.2 Verify the rungs; stair& working platform are not slippery. 2.10.3 Personal protection equipment should be worn at all times, wear safety harness if required & anchor the lanyard to safe anchorage point. 2.10.4 Verify scaffold platform / ladder is not slippery. 2.10.5 Verify proper footwear is worn and is free from mud/grease. 2.10.6 Always face the ladder when climbing up or down 2.10.7 Do not carry a load on a ladder 2.10.8 Verify that adequate illumination is provided on site. 2.10.9 Verify firm and solid landing platform / ground. | U | Cr | L |
| 3. Inspection Using Mobile Elevating Work Platform (MEWP). -Arrival on site /Pre inspection of MEWP. | 3.1 Falls on level. | 3.1.1 Verify walkway is clear. 3.1.2 Slippery surface should be avoided. 3.1.3 Wear correct footwear. | U | N | L |
| -Stepping into MEWP. | 3.2 Falls on level. | 3.2.1 Verify access gate is clear from any obstruction. 3.2.2 Verify no slippery substance on the steps and landing platform surface. 3.2.3 Verify 3 points of contact while stepping into the platform. | S | M | L |
| -Inspection on MEWP. | 3.3 Incompetent operator. | 3.3.1 Verify operator is third party certified to use the type of MEWP for which he is operating. | S | M | L |
|  | 3.4 Failure of MEWP. | 3.4..1 Verify MEWP is certified and in good condition. 3.4.2 Verify daily inspection being carried out by the operator prior to use (see the inspection report). | S | Cr | M |
|  | 3.5 Overturning / tipping over. | 3.5.1 Verify MEWP is suitable and sufficient for the ground condition. 3.5.2 Verify outriggers (if available) are properly deployed and extended and not placed near edge of excavation, manholes, etc. 3.5.3 Verify platform is not overloaded. Check Safe Working Load (SWL). 3.5.4 Verify MEWP maneuvering should be avoided while platform in the raised position unless designed to do so. 3.5.5 Verify tires are choked / blocked to prevent unintended movement. | S | Cr | M |
|  | 3.6 Falls from height. | 3.6.1 Appropriate safety harness must be used with lanyard attached to load bearing anchor. 3.6.2 Verify railings and toe boards are complete. 3.6.3 Verify gate is secured and closed. 3.6.4 Do not elevate your position by standing on the guardrail. | S | Cr | M |
|  | 3.7 Contact with fixed machinery. | 3.7.1 Verify operator is third party certified and operate the type of MEWP for which he is trained. 3.7.2 Verify operator is site oriented and familiar with the working area. 3.7.3 Verify banksman or spotter is designated to check the clearance between the equipment and the fixed machinery. 3.7.4 Verify operator safely and smoothly operates the equipment. | S | M | L |
|  | 3.8 Contact with moving vehicles. | 3.8.1 Verify work area is cordoned off around MEWP. 3.8.2 Verify competent banksman is designated. 3.8.3 Verify traffic management plan is observed and implemented. | S | Cr | M |
|  | 3.9 Adverse weather / strong wind. | 3.9.1 Verify MEWP should not operate beyond wind speed limit specified by manufacturer. 3.9.2 Work should be immediately stopped and platform to be brought down on the ground. | S | M | L |
|  | 3.10 Overhead power lines. | 3.10.1 Avoid working near overhead electrical cables. 3.10.2 Verify safe working distance is maintained from overhead live electrical cables. | S | Cr | M |
|  | 3.11 Heat Stress. | 3.11.1 Exposure under hot environment should be regulated. 3.11.2 Avoid dehydration. Drink water frequently in small amounts. 3.11.3 Wear light and comfortable clothing during hot weather. | S | M | L |
| -Getting off from MEWP. | 3.12 Falls on level. | 3.12.1 Use 3 points of contact. 3.12.2 No oil and grease on steps or shoes. 3.12.3 Use step/ladder provided. | S | M | L |
| 4. Inspection using suspended working platform -Arrival on site / Pre-inspection of suspended platform. | 4.1 Falls on level. | 4.1.1 Verify walkway is clear. 4.1.2 Slippery surface should be avoided. 4.1.3 Wear correct footwear. | U | M | L |
|  | 4.2 Falling materials. | 4.2.1 Verify contractor coordinated with building owner / other contractors in the building about the activity and the side of the building where the suspended platform is fixed has been secured. 4.2.2 Wear hardhat. | S | Cr | M |
| -Stepping into the platform. | 4.3 Falls on level. | 4.3.1 Verify access gate is clear from any material obstruction. 4.3.2 No oil/grease on shoes or steps. 4.3.3 Verify to access only in designated access point. 4.3.4 Apply 3 points of contact. | U | M | L |
| -Inspection on the platform. | 4.4 Failure of suspended platform. | 4.4.1 Verify platform is certified and in good condition. Verify daily inspection being carried out by the operator prior to use (see the inspection report). 4.4.2 Major lifting component like ropes/cables, platforms, etc. should be visually checked for defect. 4.4.3 Verify platform is not overloaded. Check SWL. A permanent and prominent sign shall be attached to the platform, stating the safe working load and the maximum operating wind speed. | S | Cr | M |
| -Inspection on the platform. (cont…) | 4.5 Incompetent operator. | 4.5.1 Verify operator is third party certified and specifically trained for the purpose. | S | M | L |
|  | 4.6 Entanglement / entrapment in moving parts. | 4.6.1 Verify guard / cover is in place for moving mechanical part. 4.6.2 Do not place body part near the movable part of the equipment. 4.6.3 Do not wear loose clothing. | S | Cr | M |
|  | 4.7Platform struck by sudden opening of the window. | 4.7.1 Verify contractor coordinated and advised building owner/occupants regarding scheduled activities and secured the side of the building properly. | S | M | L |
|  | 4.8 Heat stress. | 4.8.1 Exposure under hot environment should be regulated. 4.8.2 Wear light clothing. 4.8.3 Avoid dehydration. Drink water frequently in small amount. | S | M | L |
|  | 4.9Falls from height. | 4.9.1 Appropriate safety harness should be used and Verify it is hooked on independent lifeline. 4.9.2 Verify adequate handrails, mid-rails, and toe boards are in place. 4.9.3 Verify gate is locked and closed. | S | Cr | M |
|  | 4.10During strong wind. | 4.10.1 Work should be stopped and Verify platform on the level ground. 4.10.2 A permanent and prominent sign shall be attached to the platform, stating the safe working load and the maximum operating wind speed. | S | Cr | M |
|  | 4.11Working near live electrical cables. | 4.11.1 Verify live electrical cable is identified by contractor and properly protected. 4.11.2 Safe working distance from live electrical cable should be maintained. | S | Cr | M |
| -Exiting the Suspended Platform. | 4.1 2Falls on level. | 4.12.1 3 points of contact at all times when exiting. 4.12.2 Verify access/egress gate is not obstructed. 4.12.3 No oil/grease on shoes or steps. 4.12.4 Verify to egress from designated access point. Do NOT climb over the rails. | S | M | L |
| Equipment to be Used | Training Requirements/Competent or Qualified Personnel | Inspection Requirements |  |  |  |  |
| - Basic PPE + SafetyHarness. | 1. SH&E Site Induction. | PPE to be inspected prior to use. All Employees are to report unsafe conditions as they are observed to their immediate supervisor and a project safety committee member Effort should be made to communicate findings to contractor. |  |  |  |  |