| Activity/Work Task: Working On Or Near The Line | Overall Risk Assessment Code (RAC) (Use highest code) |  |  |  |  |  |  |  | L |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Project Location: | Risk Assessment Code (RAC) Matrix |  |  |  |  |  |  |  |  |  |
| Contract Number: | Severity | Probability |  |  |  |  |  |  |  |  |
| Date Prepared: |  | Frequent (F) | Likely (L) | Occasional (O) |  |  |  | Seldom (S) |  | Unlikely (U) |
| Prepared by (Name/Title): | Catastrophic (C) | E | E |  | H |  | H |  |  | M |
|  | Critical (Cr) | E | H |  | H |  | M |  |  | L |
| Reviewed by (Name/Title): | Marginal (M) | H | M |  | M |  | L |  |  | L |
|  | Negligible (N) | M | L |  | L |  | L |  |  | L |
| Employer/GBU: | Step 1: Review each “Hazard” with identified safety “Controls” and determine RAC (See above) Note: The RAC is determined after implementing the control for listed hazard. |  |  |  |  |  |  |  |  |  |
| Notes: (Field Notes, Review Comments, etc.) References : | P “Probability” is the likelihood to cause an incident, near miss, or accident and identified as: Frequent (F), Likely (L), Occasional (O), Seldom (S) or Unlikely (U). |  |  |  |  | RAC Chart |  |  |  |  |
|  | S “Severity” is the outcome/degree if an incident, near miss, or accident did occur and identified as: Catastrophic (C), Critical (Cr), Marginal (M), or Negligible (N) |  |  |  |  | E = Extremely High Risk |  |  |  |  |
|  |  |  |  |  |  | H = High Risk |  |  |  |  |
|  | Step 2: Identify the RAC (Probability/Severity) as E, H, M, or L for each “Hazard” on AHA. Annotate the overall highest RAC at the top of AHA. |  |  |  |  | M = Moderate Risk |  |  |  |  |
|  |  |  |  |  |  | L = Low Risk |  |  |  |  |

| Job Steps | Hazards | Controls | P | S | RAC |
| --- | --- | --- | --- | --- | --- |
| - Working On Or Near The Line | - Struck by Train – Objects Thrown from trains / Defective train parts ejected / train strikes trackside equipment which it throws up. | - All staff shall be in possession of a Personal Track Safety competence - Track Visitor Permits shall be issued for work in red and green zone, the visitor shall be briefed on the Safe system of work and accompany the supervisor/ team leader at all times - Authorized walking routes shall be used where available - Access and egress to the track shall only be made through authorized access points. - All work shall be planned in advance (except in emergencies) to Abu Dhabi Construction Code Of Practices for ‘Protection of People Working On or Near the Line’ - Walking, inspecting or working shall take place in the four-foot only where necessary at all other times it shall be undertaken from the cess - All work shall be performed by either : Use of Green Zones e.g. Safeguarded, Separated or Fenced - Lookout Operated Warning System, Tough operated warning systems, or Lookout - The Hazard Directory shall be consulted to check for Red flagged area Prohibited locations, access points, limited clearances - All Controllers of Site Safety shall provide a site specific briefing on all hazards, the safe system of work, including the position of safety and access to and from the worksite - The Sectional Appendix shall be consulted to check for line speeds, bi-directional lines, tunnels and local instructions - The Weekly Operating Notice shall be checked for single line working arrangements and the Signaler contacted to advise of work in area and to identify any alterations, such as emergency single line working - Authorized walking routes shall always be used and where possible alternative safer routes taken e.g. use of bridges and subways - High visibility clothing shall be worn at all times in accordance with the Etihad Rail Protective Equipment Policy and not be hidden by use of personal equipment - Staff shall acknowledge all warnings from lookouts, site wardens and trains and move immediately to a position of safety when a train approaches - When walking to and from worksites staff shall face oncoming traffic wherever possible, they shall not walk in the 4 foot or on sleeper ends and maintain as much distance as possible from the running lines. - Ballast & other materials shall not be allowed to pile up - All materials shall be stored securely when work has finished | S | M | L |
| - Walking and working in darkness. | - Struck by Train | - All employees shall be provided with personal lighting e.g. torch or headlamp when working in darkness or tunnels - All worksites shall be provided with suitable temporary or permanent lighting - The COSS shall brief all staff on hazards of working in darkness / tunnels and check that appropriate controls are in place - Where loading / unloading road vehicles or performing other tasks or on the road then lights on vehicles shall be used and other suitable barriers, lighting put in place - Walking or working within tunnels shall only be performed in green zones, except where permitted otherwise in local instructions - Work in tunnels shall be performed in green zones or specific red zone using Lookout Operating Warning Systems / Train Operated Warning System, etc | S | M | L |
|  | - Electrocution | - See AHA for Working adjacent to DC electrified rails Levels - See AHA for Working near electrified overhead line equipment | S | M | L |
|  | - Slips, Trips and Falls | - Tools and Materials shall be kept clear of walking routes, walkways, refuges or recesses - See AHA – Underfoot Conditions | S | M | L |
|  | - Electrical Cables | - As far as possible do not interfere with line side cables - Warning notices for High-voltage cables and markers showing location of cables shall be kept free from obstructions - No flammable materials shall be allowed to build up near cable routes and firebreaks under cables shall be kept clear | S | M | L |
|  | - Wrongly Operating a track circuit | - Metal measuring tapes or chains shall not be placed across the rails as they may operate a track circuit - No metal object shall come into contact with signaling equipment or within 300mm of an axle counter head | S | M | L |
| - Working in adverse weather conditions | - Fog/ Rain | - Where visibility is reduced the Controller of Site Safety shall assess the situation and implement green zone working, because reliance cannot be guaranteed on using lookouts - All staff shall wear high visibility clothing that is clean and not faded | S | M | L |
|  | - Sun/ Heat | - Staff shall cover up exposed skin and apply sunscreen where skin has to be exposed - An adequate supply of portable water shall be available at all times and taken on site if remote from vehicles, depots, etc - Staff shall be rotated to prevent excessive exposure especially, lookouts, site wardens and hand signalers | S | M | L |
|  | - Extreme Cold | - Staff shall be rotated to prevent excessive exposure especially, lookouts, site wardens and hand signalers - Hot drinks and facilities for keeping warm shall be available - Gloves shall be worn to protect hands | S | M | L |
|  | - High Winds | - An assessment of weather conditions shall be undertaken before work is performed, including weather forecasts to assess the effect high winds may have. - The Team Leader shall monitor and assess whether work shall continue - Work that is performed at height, or lifting loads (such as use of cranes) shall be assessed in high winds and stopped at certain wind speeds – See specific work activity risk control sheets - Inspect structures, such as scaffold, cranes, after high winds | S | M | L |
|  | - Rain Including Flooding | - An assessment of weather conditions shall be undertaken before work is performed, including weather forecasts to identify possible heavy rain - The Team Leader shall monitor and assess whether work shall continue - Work shall not occur in drains, culverts of confined spaces during and following heavy rains | S | M | L |
|  | - Thunder Storms and Lightning | - Work on electrical systems shall not be performed in thunderstorms - All staff shall take shelter inside, where possible, during thunderstorms | S | M | L |
| 4. Working in Green Zone | 4.1. Inexperienced Staff / Contractors | - New entrants and employees transferred from another area shall be placed on Safety Passport Scheme - Site briefing shall be conducted by Traffic Officer / Supervisor / Safety Officer. - Local supervisor / Traffic Officer / Safety Officer shall be appointed where possible, otherwise a site visit shall be arranged prior to or immediately before the team are briefed and commencement of work - The Site Warden shall be positioned by the Safety Officer were he feels it will be safest for him/her to perform his duties | S | M | L |
|  | - ineffective communication leading to being struck by train/ plant or machines | - Railway Communication Training - site briefing shall be conducted by Supervisor - Use railway communication, including use of phonetic alphabet - Use marker boards, possession limit boards and detonators to set up a designated safe system of work | S | M | L |
|  | - Lack of concentration due to stress / fatigue resulting in staff being struck by train | - Safety Officer Team Leader shall assess individual’s fitness to work - monitor hours worked | S | M | L |
| - Working In Red Zone | - Trains approaching from wrong direction | - Local Supervision shall be appointed where possible, otherwise a site visit shall be arranged prior to commencement of work - Sentinel Competencies e.g. Safety Officer/ Site Supervisor / Lookout shall be checked prior to work starting - Plan a safe system of work - Seek necessary authority for changes to Safe system of work hierarchy level - Safety Officer / Site Supervisor shall implement Safe System of - Safety officer / supervisor shall brief staff on safe system of work - Safety Officer shall test system of work prior to starting work and check that it protects from the approach of trains from all possible directions. - The supervisor shall not carry out work which requires him to use equipment/tools or wear PPE that reduces his ability to monitor his safe system of work to check it continues to be effective (e.g. personal use of noisy equipment etc.) - The supervisor shall check that all staff move to the designated position of safety when warned of every approaching train (this is especially important when working adjacent to crossovers and a pattern emerges of trains passing without using the crossover). - Distant/Intermediate Lookouts shall be positioned in a place of safety Site Lookouts shall be positioned by the supervisor / safety officer where he feels it will be safest for him/her to perform his/her duties - Supervisor shall contact Traffic Officer prior to gaining access to the track | S | M | L |
|  | - Failure of Lookout Operated Warning Systems / Tough operated Warning System / Automatic track warning systems | - Use competent and trained operators of equipment - Conduct pre-use check and test of equipment - A planned preventative maintenance program shall be in place - COSS shall brief staff on safe system of work in operation - COSS shall test system of work prior to authorizing work | S | M | L |
|  | - Tools / Equipment being struck by train | - Supervisor shall brief on clearing tools of the line - Supervisor shall make adequate time available within safe system of work to remove tools from the line prior to trains passing site - Tools and equipment shall be removed to a position of safety prior to trains passing site as per the Rule Book | S | M | L |
| - Cab Riding | - Distraction of Driver | - Recording devices shall not be used while in the cab | S | M | L |
|  | - Trapping of Body parts | - Hand/Foot protection shall be worn when entering/alighting from cab - When closing doors on train hands / fingers / clothing / bags shall be kept clear to prevent entrapment | S | M | L |
|  | - Slips, trips and falls from train | - Three points of contact shall be maintained with train steps / handrail - Sound footing shall be achieved when accessing and alighting the train. Do not let go of the handrail until footing is assured. - Train shall be stopped before accessing or | S | M | L |
|  | - Electrocution | - Consider DC line when entering from or alighting to cess | S | M | L |
|  | - Train Movements | - A safe system for trains on open lines shall be established when entering/alighting from cabs outside station platforms & in sidings | S | M | L |
| Equipment to be Used | Training Requirements/Competent or Qualified Personnel | Inspection Requirements |  |  |  |
| - Safety helmet - Safety Shoes - High Visibility vest - Hand gloves - Respiratory protection (As Needed) - Safety Glasses | - Basic Safety Induction - Training on safe use of PPE - Emergency Procedures | - Daily - Weekly - When Required |  |  |  |