| Activity/Work Task: Working Over Or Near Water | Overall Risk Assessment Code (RAC) (Use highest code) |  |  |  |  |  |  |  | M |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Project Location: | Risk Assessment Code (RAC) Matrix |  |  |  |  |  |  |  |  |  |
| Contract Number: | Severity | Probability |  |  |  |  |  |  |  |  |
| Date Prepared: |  | Frequent (F) | Likely (L) | Occasional (O) |  |  |  | Seldom (S) |  | Unlikely (U) |
| Prepared by (Name/Title): | Catastrophic (C) | E | E |  | H |  | H |  |  | M |
|  | Critical (Cr) | E | H |  | H |  | M |  |  | L |
| Reviewed by (Name/Title): | Marginal (M) | H | M |  | M |  | L |  |  | L |
|  | Negligible (N) | M | L |  | L |  | L |  |  | L |
| Employer/GBU: | Step 1: Review each “Hazard” with identified safety “Controls” and determine RAC (See above). The RAC is developed after correctly identifying all the hazards and fully implementing all controls. |  |  |  |  |  |  |  |  |  |
| Notes: (Field Notes, Review Comments, etc.) References : | P “Probability” is the likelihood to cause an incident, near miss, or accident and identified as: Frequent (F), Likely (L), Occasional (O), Seldom (S) or Unlikely (U). |  |  |  |  | RAC Chart |  |  |  |  |
|  | S “Severity” is the outcome/degree if an incident, near miss, or accident did occur and identified as: Catastrophic (C), Critical (Cr), Marginal (M), or Negligible (N) |  |  |  |  | E = Extremely High Risk |  |  |  |  |
|  |  |  |  |  |  | H = High Risk |  |  |  |  |
|  | Step 2: Identify the RAC (Probability/Severity) as E, H, M, or L for each “Hazard” on AHA. Annotate the overall highest RAC at the top of AHA. |  |  |  |  | M = Moderate Risk |  |  |  |  |
|  |  |  |  |  |  | L = Low Risk |  |  |  |  |

| Job Steps | Hazards | Controls | P | S | RAC |
| --- | --- | --- | --- | --- | --- |
| 1. Driving in the vicinity of water ways. | 1.1 Falls of vehicle into water. | 1.1.1 Maintain a safe distance while driving near water. 1.1.2 Park parallel to water never facing towards it. 1.1.3 Obey mandatory site speed limit. 1.1.4 Carry out pre use checks of vehicle. | U | Cr | L |
|  | 1.2 Collision with vehicle/ workers. | 1.2.1 Only employees with a valid drivers license for the type of vehicle should be allowed to drive 1.2.2 Use designated access roads only. 1.2.3 Follow defensive driving techniques. 1.2.4 Obey mandatory site speed limit. | U | Cr | L |
| 2. Arrival at site. 2. Arrival at site (Cont…) | 2.1 Falls on same level. | 2.1.1 Ensure the use of appropriate foot wear. 2.1.2 Ensure that the area is free from waste/ scrap material. 2.1.3 Use designated walk ways only. 2.1.4 Walk two (2) meters away from the water. 2.1.5 Wear a life jacket while near or over water. | U | Cr | L |
|  | 2.2 Collision with plant. 2.2 Collision with plant (Cont…) | 2.2.1 Use designated walk ways only. 2.2.2 Never walk behind any moving plant. 2.2.3 Ensure Proper lighting is in place. 2.2.4 Ensure the use of a high visibility vest. | U U | Cr Cr | L L |
| 3. Carrying out inspections within 2 (two) meters of the waters edge. | 3.1 Falls on same level. | 3.1.1 Ensure the use of appropriate foot wear. 3.1.2 Ensure that the area is free from waste/ scrap material. 3.1.3 Use designated walk ways only. 3.1.4 Ensure that edge protection is in place. 3.1.5 Wear a life jacket while near or over water. | U | Cr | L |
|  | 3.2 Fall Into Water | 3.2.1 Ensure that edge protection is in place. 3.2.2 Maintain a safe distance from edge while walking. | S | Cr | M |
|  | 3.3 Drowning. | 3.3.1 Always wear a life jacket while working over or near water. 3.3.2 Ensure that rescue/ emergency procedures are in place. 3.3.3 Ensure that floatation devices are available around the area and in good condition. 3.3.4 Always use buddy system while working over or near water. | U | Cr | L |
| 4. Transport on a shuttle boat. | 4.1 Fall into water. | 4.1.1 Always wait till the boat is secured and only then get on/ off the boat. 4.1.2 Standing on the boat while the boat is in motion is not permitted. 4.1.3 Always follow the instructions of the boat operator. 4.1.4 Life jackets must be worn while travelling on boat. 4.1.5 Ensure that the boat is not crowded/ over loaded. | S | Cr | M |
| 5. Inspection of pier/ more structure. | 5.1 Falls on same level. | 5.1.1 Ensure the use of appropriate foot wear. 5.1.2 Ensure that the area is free from waste/ scrap material. 5.1.3 Use designated walk ways only. 5.1.4 Ensure that edge protection is in place. 5.1.5 Wear a life jacket while near or over water. | U | Cr | L |
|  | 5.2 Use of Ladder. | 5.2.1 Refer to AHA 06 Use Of Ladder. |  |  |  |
|  | 5.3 Falls from height. | 5.3.1 Always maintain 3 point contact while using ladder. 5.3.2 Ensure stairs are clear from any obstruction & non slippery. 5.3.3 Ensure rungs of the ladder are clear of mud/grease & are not damaged. 5.3.4 Ensure proper footwear is worn and is free from mud/grease. 5.3.5 Ensure ladder extends 1 meter from the top landing platform. 5.3.6 Ensure ladder is safely positioned at 75 degree angle (1:4 ratio). 5.3.7 Do not carry a load on a ladder 5.3.8 Ensure that adequately illumination is provided on site. 5.3.9 Always face the ladder when climbing up or down | U | Cr | L |
|  | 5.4 Fall into water. | 5.4.1 Ensure safe access & egress to the structure in line with local legislations. 5.4.2 Never lean on guard rails. | U | Cr | L |
|  | 5.5 Drowning. | 5.5.1. Always wear a life jacket while working over or near water. 5.5.2 Ensure that rescue/ emergency procedures are in place. 5.5.3 Ensure that floatation devices are available around the area and in good condition. 5.5.4 Always use buddy system while working over or near water. | U | Cr | L |
| Equipment to be Used | Training Requirements/Competent or Qualified Personnel | Inspection Requirements |  |  |  |
| Personal Protective Equipment. | SH&E Site Induction. | PPE to be inspected monthly. All Employees are to report unsafe conditions as they are observed to their immediate supervisor and a project safety committee member. Effort should be made to communicate findings to contractor. |  |  |  |