| Activity/Work Task: Working With And Mixing Concrete | Overall Risk Assessment Code (RAC) (Use highest code) |  |  |  |  |  |  |  | L |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Project Location: | Risk Assessment Code (RAC) Matrix |  |  |  |  |  |  |  |  |  |
| Contract Number: | Severity | Probability |  |  |  |  |  |  |  |  |
| Date Prepared: |  | Frequent (F) | Likely (L) | Occasional (O) |  |  |  | Seldom (S) |  | Unlikely (U) |
| Prepared by (Name/Title): | Catastrophic (C) | E | E |  | H |  | H |  |  | M |
|  | Critical (Cr) | E | H |  | H |  | M |  |  | L |
| Reviewed by (Name/Title): | Marginal (M) | H | M |  | M |  | L |  |  | L |
|  | Negligible (N) | M | L |  | L |  | L |  |  | L |
| Employer/GBU: | Step 1: Review each “Hazard” with identified safety “Controls” and determine RAC (See above) Note: The RAC is determined after implementing the control for listed hazard. |  |  |  |  |  |  |  |  |  |
| Notes: (Field Notes, Review Comments, etc.) References : | P “Probability” is the likelihood to cause an incident, near miss, or accident and identified as: Frequent (F), Likely (L), Occasional (O), Seldom (S) or Unlikely (U). |  |  |  |  | RAC Chart |  |  |  |  |
|  | S “Severity” is the outcome/degree if an incident, near miss, or accident did occur and identified as: Catastrophic (C), Critical (Cr), Marginal (M), or Negligible (N) |  |  |  |  | E = Extremely High Risk |  |  |  |  |
|  |  |  |  |  |  | H = High Risk |  |  |  |  |
|  | Step 2: Identify the RAC (Probability/Severity) as E, H, M, or L for each “Hazard” on AHA. Annotate the overall highest RAC at the top of AHA. |  |  |  |  | M = Moderate Risk |  |  |  |  |
|  |  |  |  |  |  | L = Low Risk |  |  |  |  |

| Job Steps | Hazards | Controls | P | S | RAC |
| --- | --- | --- | --- | --- | --- |
| - Working With and Mixing Concrete | - Manual Handling | - All staff shall be trained in manual handling - All loads shall be a maximum weight of 25kg | S | M | L |
|  | - Contact with Cement and Dust | - Overalls, Eye protection and gloves shall be worn when mixing or handling cement and an eye wash station provided if running water is not available - Respiratory protective equipment (e.g. dust mask) shall be worn at all times when mixing cement - Always wash hands before eating, drinking or smoking – washing facilities shall be provided on site - Provide facilities for drying and changing clothing on site and laundry services, to prevent cement burns - Staff who are regularly exposed to cement shall have access to health surveillance, including skin checks, as detailed in the COSHH risk assessment system. | S | M | L |
| 2. Use of Concrete Mixers | - Defective Mixers, Contact with Moving Parts, Noise | - A planned preventative maintenance program shall be in place and only cement mixers below the first action noise level (80dB) supplied - Operators shall perform a pre-use check to check that the mixer is fit for purpose and within inspection dates - Cement mixers shall only be operated by trained staff and unauthorized access prevented | S | M | L |
|  | - Vehicle Driven Cement Mixers | - All movements within the worksite shall be controlled using a competent banksman and delivery drivers shall be briefed on the site safety arrangements prior to delivery and supervised at all times | S | M | L |
|  | - Vibrating Concrete | - Only vibrating pokers that have had a full Hand-Arm Vibration assessment performed shall be used and staff shall be rotated and not exceed the maximum time limit required of the Hand Arm Vibration risk assessment | S | M | L |
| Equipment to be Used | Training Requirements/Competent or Qualified Personnel | Inspection Requirements |  |  |  |
| - Safety helmet - Safety Shoes - High Visibility vest - Hand gloves - Respiratory protection (As Needed) - Safety Glasses | - Basic Safety Induction - Training on safe use of PPE - Emergency Procedures | - Daily - Weekly - When Required |  |  |  |