| Activity/Work Task: Working With On Track Equipment | Overall Risk Assessment Code (RAC) (Use highest code) |  |  |  |  |  |  |  | L |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Project Location: | Risk Assessment Code (RAC) Matrix |  |  |  |  |  |  |  |  |  |
| Contract Number: | Severity | Probability |  |  |  |  |  |  |  |  |
| Date Prepared: |  | Frequent (F) | Likely (L) | Occasional (O) |  |  |  | Seldom (S) |  | Unlikely (U) |
| Prepared by (Name/Title): | Catastrophic (C) | E | E |  | H |  | H |  |  | M |
|  | Critical (Cr) | E | H |  | H |  | M |  |  | L |
| Reviewed by (Name/Title): | Marginal (M) | H | M |  | M |  | L |  |  | L |
|  | Negligible (N) | M | L |  | L |  | L |  |  | L |
| Employer/GBU: | Step 1: Review each “Hazard” with identified safety “Controls” and determine RAC (See above) Note: The RAC is determined after implementing the control for listed hazard. |  |  |  |  |  |  |  |  |  |
| Notes: (Field Notes, Review Comments, etc.) References : | P “Probability” is the likelihood to cause an incident, near miss, or accident and identified as: Frequent (F), Likely (L), Occasional (O), Seldom (S) or Unlikely (U). |  |  |  |  | RAC Chart |  |  |  |  |
|  | S “Severity” is the outcome/degree if an incident, near miss, or accident did occur and identified as: Catastrophic (C), Critical (Cr), Marginal (M), or Negligible (N) |  |  |  |  | E = Extremely High Risk |  |  |  |  |
|  |  |  |  |  |  | H = High Risk |  |  |  |  |
|  | Step 2: Identify the RAC (Probability/Severity) as E, H, M, or L for each “Hazard” on AHA. Annotate the overall highest RAC at the top of AHA. |  |  |  |  | M = Moderate Risk |  |  |  |  |
|  |  |  |  |  |  | L = Low Risk |  |  |  |  |

| Job Steps | Hazards | Controls | P | S | RAC |
| --- | --- | --- | --- | --- | --- |
| - Working With On Track Equipment | - Struck By Machine | - The traffic officer, supervisor and On Track Machine driver shall reach a clear understanding regarding movements of machine - At each marker board only the Traffic Officer shall give permission for the machine to enter the worksite - The supervisor shall establish where the On Track Machine is required to set up for work - The safety Officer shall establish and brief to all staff the safe system of work to be adopted including - Isolation limits obtained - Protection arrangements for adjacent lines - An exclusion zone of 3 meters for staff not working on the machine - Any changes to the arrangements for safe working shall be briefed to all staff prior to the changes being implemented - Machine horn to be sounded prior to movement and when changing direction of travel | S | M | L |
|  | - Struck By Train | - See AHA Work On or Near the Line - The Safety Officer shall brief to all staff on the lines open to traffic and the line speed – and the position of safety to be used. - A lookout shall be appointed, if necessary, to maintain staff safety | S | M | L |
|  | - Noise | - Ear Protection shall be worn within the hearing protection zone | S | M | L |
|  | - Dust and Fumes | - Supervisor shall brief all staff on the safe system of work for the work within the tunnel including the limits of the exclusion zone - Supervisor shall clear all staff from within the tunnel prior to any change of direction of the machine - No machine movements shall commence until the Traffic Officer has given permission - Prior to movement the machine operator shall sound the horn - The machine operator shall remain with the machine at all times - Staff shall wear appropriate PPE ( Masks/ Ear Defenders etc) and remain outside the exclusion zone | S | M | L |
|  | - Dust and Silicates | - Where the ballast is disturbed staff in the vicinity shall wear dust masks and eye protection. | S | M | L |
|  | - Lasers | - Lasers on machines shall be screened/shielded | S | M | L |
|  | - Work In Darkness | - Adequate lighting shall be provided Staff shall be issued with Cap/hand lamps - See AHA Work On or Near the Line | S | M | L |
|  | - Poor Visibility (Dust/ Smoke – Slips Trips and Falls) | - Excessive dust shall be reported to the machine operator via the supervisor and safety officer - Dampen down work site, if possible, to reduce dust levels | S | M | L |
|  | - Fire From Sparks Emitted From a Rail Grinder. | - Site shall be cleared of flammable material prior to work commencing. - Fire extinguishers shall be available on site - Spark guards shall be fitted and checked prior to work commencing | S | M | L |
| Equipment to be Used | Training Requirements/Competent or Qualified Personnel | Inspection Requirements |  |  |  |
| - Safety helmet - Safety Shoes - High Visibility vest - Hand gloves - Respiratory protection (As Needed) - Safety Glasses | - Basic Safety Induction - Training on safe use of PPE - Emergency Procedures | - Daily - Weekly - When Required |  |  |  |