| Activity/Work Task: Working With Or Near Mobile Plant | Overall Risk Assessment Code (RAC) (Use highest code) |  |  |  |  |  |  |  | L |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Project Location: | Risk Assessment Code (RAC) Matrix |  |  |  |  |  |  |  |  |  |
| Contract Number: | Severity | Probability |  |  |  |  |  |  |  |  |
| Date Prepared: |  | Frequent (F) | Likely (L) | Occasional (O) |  |  |  | Seldom (S) |  | Unlikely (U) |
| Prepared by (Name/Title): | Catastrophic (C) | E | E |  | H |  | H |  |  | M |
|  | Critical (Cr) | E | H |  | H |  | M |  |  | L |
| Reviewed by (Name/Title): | Marginal (M) | H | M |  | M |  | L |  |  | L |
|  | Negligible (N) | M | L |  | L |  | L |  |  | L |
| Employer/GBU: | Step 1: Review each “Hazard” with identified safety “Controls” and determine RAC (See above) Note: The RAC is determined after implementing the control for listed hazard. |  |  |  |  |  |  |  |  |  |
| Notes: (Field Notes, Review Comments, etc.) References : | P “Probability” is the likelihood to cause an incident, near miss, or accident and identified as: Frequent (F), Likely (L), Occasional (O), Seldom (S) or Unlikely (U). |  |  |  |  | RAC Chart |  |  |  |  |
|  | S “Severity” is the outcome/degree if an incident, near miss, or accident did occur and identified as: Catastrophic (C), Critical (Cr), Marginal (M), or Negligible (N) |  |  |  |  | E = Extremely High Risk |  |  |  |  |
|  |  |  |  |  |  | H = High Risk |  |  |  |  |
|  | Step 2: Identify the RAC (Probability/Severity) as E, H, M, or L for each “Hazard” on AHA. Annotate the overall highest RAC at the top of AHA. |  |  |  |  | M = Moderate Risk |  |  |  |  |
|  |  |  |  |  |  | L = Low Risk |  |  |  |  |

| Job Steps | Hazards | Controls | P | S | RAC |
| --- | --- | --- | --- | --- | --- |
| - Working With Or Near Mobile Plant | - Struck By Machine/ Entrapment Or Crush | - The Supervisor/ Team Leader shall establish and brief to all staff the safe system of work to be adopted include - Isolation limits obtained - Protection arrangements for adjacent lines - An exclusion zone of 3 meters for staff not working on the machine - Horn to be sounded prior to movement and when changing direction of travel - Any changes to the arrangements for safe working shall be briefed to all staff prior to the changes being implemented - Work shall be planned so that staff remain visible to the Machine Operator - Staff shall not approach machines where there is limited clearance unless the machine and all work movements are stopped and isolated to prevent accidental movement. - Staff shall not work under booms, platforms or raised loads - Staff shall not make adjustments or attempt to clear obstructions until the machine has been stopped, the brake applied and the engine switched off. - Staff shall not perform any actions between two machines until they are both stopped, brakes applied and the engines switched off. | S | M | L |
|  | - Work On Or Near The Line | - See AHA For Work On or Near the Line - The Supervisor / Team Leader shall brief to all staff the lines open to traffic and the line speed – and the position of safety to be used. - A Site Warden shall be appointed to maintain staff safety | S | M | L |
|  | - Noise | - Ear Protection shall be worn within the hearing protection zone | S | M | L |
|  | - Dust And Silicates And Fumes | - In dry conditions, where there is a risk of ballast dust, face mask and eye protection shall be worn - The Supervisor / Team Leader shall brief all staff on the SSOW for the work within the tunnel including the limits of the exclusion zone - The Supervisor / Team Leader shall clear all staff from within the tunnel prior to any change of direction of the machine - No machine movements shall commence until the Supervisor / Team Leader has given permission Prior to movement the machine operator shall sound the horn Machine - The machine operator shall remain with the machine at all times Operator - Staff shall wear appropriate PPE ( Masks/ Ear Defenders etc)All Staff and remain outside the exclusion zone | S | M | L |
|  | - Work In Darkness | - Adequate lighting shall be provided - Staff shall be issued with Cap/hand lamps - See AHA Work On or Near the Line | S | M | L |
|  | - Communication | - A method of communication shall be established between the Supervisor / Team Leader and machine operator either by radio or hand signals |  |  |  |
| Equipment to be Used | Training Requirements/Competent or Qualified Personnel | Inspection Requirements |  |  |  |
| - Safety helmet - Safety Shoes - High Visibility vest - Hand gloves - Respiratory protection (As Needed) - Safety Glasses | - Basic Safety Induction - Training on safe use of PPE - Emergency Procedures | - Daily - Weekly - When Required |  |  |  |