| Activity/Work Task: Working With Rail – Jointed, Check and CWR | Overall Risk Assessment Code (RAC) (Use highest code) |  |  |  |  |  |  |  | L |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Project Location: | Risk Assessment Code (RAC) Matrix |  |  |  |  |  |  |  |  |  |
| Contract Number: | Severity | Probability |  |  |  |  |  |  |  |  |
| Date Prepared: |  | Frequent (F) | Likely (L) | Occasional (O) |  |  |  | Seldom (S) |  | Unlikely (U) |
| Prepared by (Name/Title): | Catastrophic (C) | E | E |  | H |  | H |  |  | M |
|  | Critical (Cr) | E | H |  | H |  | M |  |  | L |
| Reviewed by (Name/Title): | Marginal (M) | H | M |  | M |  | L |  |  | L |
|  | Negligible (N) | M | L |  | L |  | L |  |  | L |
| Employer/GBU: | Step 1: Review each “Hazard” with identified safety “Controls” and determine RAC (See above) Note: The RAC is determined after implementing the control for listed hazard. |  |  |  |  |  |  |  |  |  |
| Notes: (Field Notes, Review Comments, etc.) References : | P “Probability” is the likelihood to cause an incident, near miss, or accident and identified as: Frequent (F), Likely (L), Occasional (O), Seldom (S) or Unlikely (U). |  |  |  |  | RAC Chart |  |  |  |  |
|  | S “Severity” is the outcome/degree if an incident, near miss, or accident did occur and identified as: Catastrophic (C), Critical (Cr), Marginal (M), or Negligible (N) |  |  |  |  | E = Extremely High Risk |  |  |  |  |
|  |  |  |  |  |  | H = High Risk |  |  |  |  |
|  | Step 2: Identify the RAC (Probability/Severity) as E, H, M, or L for each “Hazard” on AHA. Annotate the overall highest RAC at the top of AHA. |  |  |  |  | M = Moderate Risk |  |  |  |  |
|  |  |  |  |  |  | L = Low Risk |  |  |  |  |

| Job Steps | Hazards | Controls | P | S | RAC |
| --- | --- | --- | --- | --- | --- |
| - Working With Rail - Joined, Check and CWR | - Electrocution | - See AHA for Working Adjacent to DC Electrified Rails - See AHA for Working Near Electrical Overhead Line Equipment - When required, continuity bonds are fitted when working on the traction return rail in AC areas | S | M | L |
|  | - Use Of Jacks | - Inspect equipment prior to use for good condition - Operate jack by adopting the correct stance to the jacks position - A firm base for the jack shall be established - Jack shall have been maintained and shall be within its service date - Body parts / clothing and team members shall be clear when lowering - Do not stand on jacked rail - See AHA for Use of Jacks | S | M | L |
|  | - Use Of Torque | - The torque wrench shall be set to the correct setting for the bolts concerned prior to use - Torque wrench shall be in good condition and within its calibration date before use - The correct stance shall be taken when using the torque wrench | S | M | L |
|  | - Use of RRV | - See AHA for Transit of On Track Plant with/without a Machine Controller present | S | M | L |
|  | - Use of Abrasive Wheels | - Plant shall have been maintained and be within its inspection date - Only trained and competent operators shall operate, change and mount abrasive wheels - See AHA for Use of Abrasive Wheels for HAV information and operation | S | M | L |
|  | - Manual Handling of rail | - Team Lifting shall be coordinated by one person - Scrap shall be cut into smaller lengths for disposal - Mechanized means of handling shall be used wherever possible - Use mechanical lifting devices as per training e.g. Iron men and Ponies | S | M | L |
|  | - Use of Turning Bar | - All turning bars shall be checked against an approved gauge and that they are in good condition prior to use NOTE:- Bullhead and Flat bottom turning bars are different - Timber blocks/battens shall be used to protect cables and other line side equipment. - Only trained/competent staff shall undertake rail turning activities - One person shall be appointed to control the operation - Staff not involved shall be kept clear of the operation - Underfoot conditions shall be checked and obstructions, loose material shall be cleared. - Body parts / clothing and team members shall be clear when lowering/moving - Ballast shall be cleared and level to prevent sliding/rolling of the rail - Gloves shall be worn when barring rail - Care shall be taken if turning bars are used with a pulling motion rather than pushing. Pulling puts the operative in direct line of striking if a rail tips over | S | M | L |
|  | - Grinding / Drilling Rail | - Eye and hand protection shall be worn for protection from burrs - Allow drill bits / grinding wheels to cool before handling - See AHA for Use of Abrasive Wheels, - See AHA for Use of Rail Grinders - See AHA for Use of Rail/Sleeper Drill for controls and HAV information - Only trained competent staff to use equipment | S | M | L |
|  | - Kango Packing | - Eye Protection shall be worn - Ear Protection shall be worn - Warning System (Touch Lookout) shall be in place | S | M | L |
|  | - Mechanical Packing | - Eye Protection shall be worn - Ear Protection shall be worn - Warning System (ie Touch Lookout)shall be in place - See AHA for Use of Cobra/HILTI Tamping Hammers | S | M | L |
|  | - Trolley | - See AHA for Use of Manual Trolleys/Rail Skate/Scooter - Correct RIMMINI plan shall be used to allow use | S | M | L |
|  | - Dust and Silicates | - COSHH Assessments and controls shall be available | S | M | L |
|  | - Fire | - Remove flammable debris from site - Dry Powder Fire extinguisher shall be available at site | S | M | L |
|  | - Welding | - See AHA for Aluminothermic Welding - See AHA for Electric Arc Welding , - See AHA for Erection, Dismantling and Use of Welding Tents/Umbrellas & Support Clamps | S | M | L |
|  | - Unplanned Rail Move | - Be alert to risk of rail moving or tipping when unclipping i.e. tight curve | S | M | L |
|  | - Cold Bolt Expansion Equipment | - See AHA for Use of Cold Bolt Expansion Equipment for controls and HAV information - Only trained competent staff to use equipment | S | M | L |
|  | - Fluid and Lubricants | - COSHH Assessments and controls shall be available | S | M | L |
|  | - Use of Impact Wrench | - Check that the Impact wrench is in good condition and within its calibration date before use - Check that the correct stance is taken when using the impact wrench - Only trained competent staff shall use the equipment - Fingers/loose clothing shall be kept clear of fishplate, bolts and rail. - If ANY malfunction of the equipment occurs stop using the equipment, return as faulty and change for a new wrench. - See AHA for Use of Impact Wrench for HAV information and operation Ear protection shall be worn | S | M | L |
| Equipment to be Used | Training Requirements/Competent or Qualified Personnel | Inspection Requirements |  |  |  |
| - Safety helmet - Safety Shoes - High Visibility vest - Safety Glasses - Hand Gloves - Hearing Protection - Welding Mask - Flame Resistant Over All - Dust Mask | - Basic Safety Induction - Training on safe use of PPE - Emergency Procedures | - Daily - Weekly - When Required |  |  |  |