| Activity/Work Task: Working at Height including Bridge Deck | Overall Risk Assessment Code (RAC) (Use highest code) |  |  |  |  |  |  |  | L |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Project Location: | Risk Assessment Code (RAC) Matrix |  |  |  |  |  |  |  |  |  |
| Contract Number: | Severity | Probability |  |  |  |  |  |  |  |  |
| Date Prepared: |  | Frequent (F) | Likely (L) | Occasional (O) |  |  |  | Seldom (S) |  | Unlikely (U) |
| Prepared by (Name/Title): | Catastrophic (C) | E | E |  | H |  | H |  |  | M |
|  | Critical (Cr) | E | H |  | H |  | M |  |  | L |
| Reviewed by (Name/Title): | Marginal (M) | H | M |  | M |  | L |  |  | L |
|  | Negligible (N) | M | L |  | L |  | L |  |  | L |
| Employer/GBU: | Step 1: Review each “Hazard” with identified safety “Controls” and determine RAC (See above) Note: The RAC is determined after implementing the control for listed hazard. |  |  |  |  |  |  |  |  |  |
| Notes: (Field Notes, Review Comments, etc.) References : | P “Probability” is the likelihood to cause an incident, near miss, or accident and identified as: Frequent (F), Likely (L), Occasional (O), Seldom (S) or Unlikely (U). |  |  |  |  | RAC Chart |  |  |  |  |
|  | S “Severity” is the outcome/degree if an incident, near miss, or accident did occur and identified as: Catastrophic (C), Critical (Cr), Marginal (M), or Negligible (N) |  |  |  |  | E = Extremely High Risk |  |  |  |  |
|  |  |  |  |  |  | H = High Risk |  |  |  |  |
|  | Step 2: Identify the RAC (Probability/Severity) as E, H, M, or L for each “Hazard” on AHA. Annotate the overall highest RAC at the top of AHA. |  |  |  |  | M = Moderate Risk |  |  |  |  |
|  |  |  |  |  |  | L = Low Risk |  |  |  |  |

| Job Steps | Hazards | Controls | P | S | RAC |
| --- | --- | --- | --- | --- | --- |
| Working at Height including Bridge Deck | 1.1 Slips/Trips | 1.1.1 Wear appropriate foot wear. 1.1.2 Do not step over tools or material. 1.1.3 Use designated walk ways/ cross over’s. 1.1.4 Obey all mandatory signs. 1.1.5 Stay out of cordoned areas. 1.1.6 Keep hands out of pockets & free of objects. 1.1.7 Do not walk on slippery surfaces. 1.1.8 Verify adequate lighting is available for work in dark areas or at night. | S | M | L |
|  | 1.2 Unprotected leading edge/Opening | 1.2.1 Verify proper covering is in place 1.2.2 pay attention to all signs 1.2.3 do not enter cordoned off areas 1.2.4 Verify proper edge protection is in place 1.2.5 keep as far away from edge as work will allow | S | M | L |
|  | 1.3 Drop material or equipment | 1.3.1 Verify toe boards are in place 1.3.2 Verify safety nets are in place 1.3.3 Verify tools have a lanyard attached properly | S | M | L |
| Equipment to be Used | Training Requirements/Competent or Qualified Personnel | Inspection Requirements |  |  |  |
| Hard Hat, Safety Glasses, Class 2 Reflective Vest & Safety footwear, gloves. | - AHA’s for each task and element of the Man Camp operation - 1st Aid/CRP training - Safety Orientation | All Employees are to report unsafe conditions as they are observed to their immediate supervisor and a project safety committee member. Effort should be made to communicate findings to contractor. |  |  |  |