| Activity/Work Task: Working from an aerial lift or elevated work platform | Overall Risk Assessment Code (RAC) (Use highest code) |  |  |  |  |  |  |  | L |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Project Location: | Risk Assessment Code (RAC) Matrix |  |  |  |  |  |  |  |  |  |
| Contract Number: | Severity | Probability |  |  |  |  |  |  |  |  |
| Date Prepared: |  | Frequent (F) | Likely (L) | Occasional (O) |  |  |  | Seldom (S) |  | Unlikely (U) |
| Prepared by (Name/Title): | Catastrophic (C) | E | E |  | H |  | H |  |  | M |
|  | Critical (Cr) | E | H |  | H |  | M |  |  | L |
| Reviewed by (Name/Title): | Marginal (M) | H | M |  | M |  | L |  |  | L |
|  | Negligible (N) | M | L |  | L |  | L |  |  | L |
| Employer/GBU: | Step 1: Review each “Hazard” with identified safety “Controls” and determine RAC (See above) Note: The RAC is determined after implementing the control for listed hazard. |  |  |  |  |  |  |  |  |  |
| Notes: (Field Notes, Review Comments, etc.) References : | P “Probability” is the likelihood to cause an incident, near miss, or accident and identified as: Frequent (F), Likely (L), Occasional (O), Seldom (S) or Unlikely (U). |  |  |  |  | RAC Chart |  |  |  |  |
|  | S “Severity” is the outcome/degree if an incident, near miss, or accident did occur and identified as: Catastrophic (C), Critical (Cr), Marginal (M), or Negligible (N) |  |  |  |  | E = Extremely High Risk |  |  |  |  |
|  |  |  |  |  |  | H = High Risk |  |  |  |  |
|  | Step 2: Identify the RAC (Probability/Severity) as E, H, M, or L for each “Hazard” on AHA. Annotate the overall highest RAC at the top of AHA. |  |  |  |  | M = Moderate Risk |  |  |  |  |
|  |  |  |  |  |  | L = Low Risk |  |  |  |  |

| Job Steps | Hazards | Controls | P | S | RAC |
| --- | --- | --- | --- | --- | --- |
| - Working from an aerial lift or elevated work platform | - Mechanical Failure | - Daily Inspection of the MEWP must be done and recorded - Only well maintained equipment will be used on site - Regular maintenance of MEWP will be done - One operator will always be ready at the bottom of the MEWP to operate from ground level in case the controls on the platform are not working | S | M | L |
|  | - Crushing | - Only certified workers will be allowed to use MEWP - Driver will make sure that no one is in the path way of the MEWP prior to moving it - Operator will at all times keep all body parts within the MEWP - Operator will move the MEWP according to site speed limit | S | M | L |
|  | - Fall From height | - Employees who are in the MEWP must wear a full body safety harness and be tied off at all times - No employee in the MEWP will be allowed to move any body part out of the MEWP - Only employees who are trained in working at height and working on MEWP will be allowed to work on MEWP | S | M | L |
|  | - Equipment topple | - MEWP will be allowed to work only on flat and even surface. - The MEWP will not be overloaded at any given time. - The MEWP will not stop or stand or ride over any hollow surfaces. | S | M | L |
|  | - Obstruction/ Utilities | - MEWP will go around the area that is obstructed by any utility - Stopping or passing over any utility will not be allowed | S | M | L |
|  | - High Noise Level | - All MEWP operators and surrounding employees must wear adequate hearing protection | S | M | L |
| Equipment to be Used | Training Requirements/Competent or Qualified Personnel | Inspection Requirements |  |  |  |
| Hard Hat, Safety Glasses, Class 2 Reflective Vest & Safety footwear, gloves, Full Body Safety Harness | - AHA training - Safety Orientation - Fall protection training. - Aerial lift or elevated work platform training | - Daily Inspection of Aerial Lift or Elevated Work Platform. |  |  |  |