| Activity/Work Task: Working on Bridge Deck Edge | Overall Risk Assessment Code (RAC) (Use highest code) |  |  |  |  |  |  |  | M |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Project Location: | Risk Assessment Code (RAC) Matrix |  |  |  |  |  |  |  |  |  |
| Contract Number: | Severity | Probability |  |  |  |  |  |  |  |  |
| Date Prepared: |  | Frequent (F) | Likely (L) | Occasional (O) |  |  |  | Seldom (S) |  | Unlikely (U) |
| Prepared by (Name/Title): | Catastrophic (C) | E | E |  | H |  | H |  |  | M |
|  | Critical (Cr) | E | H |  | H |  | M |  |  | L |
| Reviewed by (Name/Title): | Marginal (M) | H | M |  | M |  | L |  |  | L |
|  | Negligible (N) | M | L |  | L |  | L |  |  | L |
| Employer/GBU: | Step 1: Review each “Hazard” with identified safety “Controls” and determine RAC (See above) Note: The RAC is determined after implementing the control for listed hazard. |  |  |  |  |  |  |  |  |  |
| Notes: (Field Notes, Review Comments, etc.) References : | P “Probability” is the likelihood to cause an incident, near miss, or accident and identified as: Frequent (F), Likely (L), Occasional (O), Seldom (S) or Unlikely (U). |  |  |  |  | RAC Chart |  |  |  |  |
|  | S “Severity” is the outcome/degree if an incident, near miss, or accident did occur and identified as: Catastrophic (C), Critical (Cr), Marginal (M), or Negligible (N) |  |  |  |  | E = Extremely High Risk |  |  |  |  |
|  |  |  |  |  |  | H = High Risk |  |  |  |  |
|  | Step 2: Identify the RAC (Probability/Severity) as E, H, M, or L for each “Hazard” on AHA. Annotate the overall highest RAC at the top of AHA. |  |  |  |  | M = Moderate Risk |  |  |  |  |
|  |  |  |  |  |  | L = Low Risk |  |  |  |  |

| Job Steps | Hazards | Controls | P | S | RAC |
| --- | --- | --- | --- | --- | --- |
| 1. Movement Across the Site. | 1.1 Unfamiliar with site layout and emergency plans. | 1.1.1 SH&E induction prior to any activity on site | S | M | L |
|  | 1.2 Passing near Moving Plant and Vehicles. | 1.2.1 Wear high-visibility vest 1.2.2 Use designated walkways 1.2.3 Do not stay behind plant/vehicles 1.2.4 Look out for moving plants and vehicles | S | M | L |
|  | 1.3 Passing in a Dusty area, dust from activities. | 1.3.1 Wear appropriate dust mask. 1.3.2 Maintain good visibility before any movement on site. | S | M | L |
|  | 1.4 Walking in extreme temperatures/weather conditions | 1.4.1 Check for humidity/temperature outside before moving across site. 1.4.2 Wear correct clothing and PPE. 1.4.3 Reduce exposure. 1.4.4 Implement work rotations. 1.4.5 Stay hydrated during hot weather condition. 1.4.6 Use skin protection like sun block, etc.. during hot weather conditions. | S | M | L |
| 2. Accessing the Bridge Deck via external staircase. | 2.1 Collapse of Staircase. | 2.1.1 Verify the staircase is correctly constructed and where required has the necessary Scaffold Tag in place. | S | M | L |
|  | 2.2 Slip Trip or Fall on the staircase. | 2.2.1 Check that the treads of the staircase are free from mud, oil or loose gravel that may cause a slip or fall. 2.2.2 Check that the soles of your shoes are free from mud, oil or grease that may cause a slip or fall. 2.2.3 Use handrails where provided. 2.2.4 Only climb one stair at a time and do not rush. | O | M | M |
| 3. Proceeding to area of work. | 3.1 Slips Trips or Falls on the level. | 3.1.1 Use designated walkways, follow pedestrian signage. 3.1.2 Wear correct safety shoes avoid mud, oil grease or water on the floor where possible. 3.1.3 Walk do not run. 3.1.4 Pay attention to where you are walking, look ahead and plan your route. | S | M | L |
|  | 3.2 Struck by passing plant | 3.2.1 Use designated walkways, follow pedestrian signage. 3.2.2 Pay attention to traffic marshals and adhere to their instructions. 3.2.3 Wear a high visibility vest and stay in areas of good light. 3.2.4 Never walk behind vehicles or in the driver/operators blind spot. | S | M | L |
| 4. Accessing the Bridge Deck Edge. | 4.1 Slip Trip or Fall stepping over rebar/safety cable | 4.1.1 Verify attached to the safety lines by harness lanyards and that lanyards are out of the way. 4.1.2 Verify that a good hand hold is taken when stepping over. 4.1.3 Do not hold or try to carry anything when stepping over. | S | M | L |
| 5. Working on the Bridge Deck Edge. | 5.1 Falls from height | 5.1.1 All work to be controlled by a Permit. 5.1.2 All persons must be attached by a minimum of one lanyard from their safety harness at all times. 5.1.3 No work will be carried out in winds greater than 40 KPH. 5.1.4 Where possible body positioning shall be away from the edge of the bridge. 5.1.5 No work will be carried out with workers backs to the edge; a side on position must be adopted. | S | Cr | M |
|  | 5.2 Falling Material. | 5.2.1 Only material that is in actual use is to be passed over the green netting and safety cable. 5.2.2 All hand tools shall be connected to a safety lanyard. 5.2.3 No material will at any time be left unattended. 5.2.4 No material will be stored on the bridge edge at any time. | S | Cr | M |
|  | 5.3 Suffering vertigo or feelings of unease at the height. | 5.3.1 Only persons who are physically and mentally capable shall carry out work. 5.3.2 No lone working on the Bridge edge is to be allowed. 5.3.3. There shall be a minimum of 3 persons in the group working on the bridge edge; the 3 rd person must also have a harness on, even if they are not on the edge in case of emergency. | S | M | L |
| Equipment to be Used | Training Requirements/Competent or Qualified Personnel | Inspection Requirements |  |  |  |
| Personal Protective Equipment (PPE). Site minimum requirements - hard hat, safety shoes, hi visibility jacket | 1. SH&E Site Induction. 2. Working on Bridge Deck/Permit system | 1. PPE to be inspected monthly. 2. Verify pre use inspection of safety harness, safety cable and anchor points. 3. All Employees are to report unsafe conditions as they are observed to their immediate supervisor and a project safety committee member. 4. Effort should be made to communicate findings to contractor. |  |  |  |