| Activity/Work Task: Working with Ballast-Regular, Glue, Shoulder Clean, Contaminate and wet beds | Overall Risk Assessment Code (RAC) (Use highest code) |  |  |  |  |  |  |  | L |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Project Location: | Risk Assessment Code (RAC) Matrix |  |  |  |  |  |  |  |  |  |
| Contract Number: | Severity | Probability |  |  |  |  |  |  |  |  |
| Date Prepared: |  | Frequent (F) | Likely (L) | Occasional (O) |  |  |  | Seldom (S) |  | Unlikely (U) |
| Prepared by (Name/Title): | Catastrophic (C) | E | E |  | H |  | H |  |  | M |
|  | Critical (Cr) | E | H |  | H |  | M |  |  | L |
| Reviewed by (Name/Title): | Marginal (M) | H | M |  | M |  | L |  |  | L |
|  | Negligible (N) | M | L |  | L |  | L |  |  | L |
| Employer/GBU: | Step 1: Review each “Hazard” with identified safety “Controls” and determine RAC (See above) Note: The RAC is determined after implementing the control for listed hazard. |  |  |  |  |  |  |  |  |  |
| Notes: (Field Notes, Review Comments, etc.) References : | P “Probability” is the likelihood to cause an incident, near miss, or accident and identified as: Frequent (F), Likely (L), Occasional (O), Seldom (S) or Unlikely (U). |  |  |  |  | RAC Chart |  |  |  |  |
|  | S “Severity” is the outcome/degree if an incident, near miss, or accident did occur and identified as: Catastrophic (C), Critical (Cr), Marginal (M), or Negligible (N) |  |  |  |  | E = Extremely High Risk |  |  |  |  |
|  |  |  |  |  |  | H = High Risk |  |  |  |  |
|  | Step 2: Identify the RAC (Probability/Severity) as E, H, M, or L for each “Hazard” on AHA. Annotate the overall highest RAC at the top of AHA. |  |  |  |  | M = Moderate Risk |  |  |  |  |
|  |  |  |  |  |  | L = Low Risk |  |  |  |  |

| Job Steps | Hazards | Controls | P | S | RAC |
| --- | --- | --- | --- | --- | --- |
| - Working with Ballast – Regular, Glue, Shoulder Clean, Contaminate and Wet Beds | - Electrocution | - See AHA Working adjacent to DC electrified rails risk - See AHA Working near electrical overhead line equipment - Consider Proximity to DC lines when using gauges / hand tools - Only Insulated tools must be used - Shields shall be used where ever required - All tools and yourself must maintain minimum safe distance from electrical lines | S | M | L |
|  | - Struck Bu OTP (On Track Plant) | - All Movements shall be controlled by the Traffic Officer and clear communication shall be established between the Engineering Supervisor and all relevant parties. - Safe system shall be set up and communicated to allow access and egress to the machine. | S | M | L |
|  | - Use Of OTP | - Competent operator and machine controller shall be appointed. - The pre use checks shall have been carried out. - Prior to movement of any OTP the horn shall be sounded - All lifts shall be planned controlled and the crane controller shall be appointed. - See AHA Working with or near mobile plant. - All machines shall be authorized by the mechanic controller. | S | M | L |
|  | - Use of Small Plant | - See AHA for Use Of Small Plant | S | M | L |
|  | - Ballast Dust | - In dry conditions where there is a risk of ballast dust, face mask and eye protection must be worn. | S | M | L |
|  | - Ballast Glue | - COSHH assessment and controls shall be available - Use suitable hand cleaner prior to eating, drinking or smoking. | S | M | L |
| Equipment to be Used | Training Requirements/Competent or Qualified Personnel | Inspection Requirements |  |  |  |
| - Safety helmet - Safety Shoes - High Visibility vest - Hand gloves - Eye protection - Hearing protection - Dust mask | - Basic Safety Induction - Training on safe use of PPE - Emergency Procedures | - Daily - Weekly |  |  |  |